| TASK | Angle Grinder |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☒ | Heat / cold | ☒ | Electricity | ☒ |
|  | Cuts / laceration | ☒ | Dust | ☒ | Rollover | ☐ |
|  | Pinch / crush | ☒ | Noise / vibration | ☒ | Plant interaction | ☐ |
|  | High pressure | ☐ | Other: | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Check for visible defects or damage. - Check cord for visible defects or damage and tested-and-tagged. - Check guards are in place. - Confirm equipment is switched “off” before plugging in. - Ensure appropriate PPE is worn. - Tie back loose clothing, jewellery or hair. - Ensure other workers are not in the area. - Ensure flammables are removed from the area. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Firmly grip the angle grinder with both hands, placing one hand on the handle and the other on the body of the tool. - Maintain a stable stance and position your body to the side of the grinder, away from the grinding wheel's path. - Power on the grinder only after completing prestart checks and ensuring a secure grip. - Allow the grinder to reach its full speed before initiating contact with the workpiece. - Approach the workpiece at a slight angle to avoid kickback. - Do not force the grinder; let it work at its own pace. - Keep a firm grip and maintain control of the tool at all times. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off the angle grinder and wait for the grinding wheel to come to a complete stop before setting the tool down. - Disconnect the grinder from the power source. - Store the angle grinder in a secure and dry location, away from direct sunlight and moisture. - Inspect the grinder for any damage or signs of wear. - Clean the tool, removing any debris or residue from the grinding wheel and body. |  |  |  |  |  |  |