| TASK | Backhoe |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☐ | Dust | ☒ | Rollover | ☒ |
|  | Pinch / crush | ☒ | Noise / vibration | ☐ | Plant interaction | ☒ |
|  | High pressure | ☐ | Other: | ☐ | Chemical | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Walk around to inspect for visible damage or leaks on the backhoe and attachments. - Complete all pre-start checks as per manufacturer’s guidelines. - Inspect attachments for wear or breakage. - Test all control functions in the operator's cabin to verify they are working correctly. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Ensure personnel are competent and authorised. - Ensure personnel are fit for duty and not impaired by drugs or alcohol. - Adjust the seat and fasten the seatbelt before starting the engine. - Do not use a mobile phone while operating. - Start the engine and monitor for any abnormal noise or warning signals. - Perform a control sweep to ensure smooth operation of the arm, boom, bucket, and stabilisers. - Operate at a safe speed and maintain exclusion zones around people, powerlines, and other hazards (use spotters if necessary). - Beware of uneven or sloping ground. - Use the backhoe's stabilisers on solid ground before beginning excavation to ensure stability. - During excavation, be mindful of the swing radius to avoid hitting nearby objects or utilities. - Continually monitor the area for personnel and obstacles while operating the backhoe. - Never carry people or lift loads over people. - Use only suitable lifting gear and attachments. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Clean the bucket and other attachments of any dirt or debris. - Return the backhoe arm to a safe, rest position and lower all stabilisers. - Turn off the engine and remove the key before exiting the operator's cabin. - Log the hours of operation and any maintenance issues noted during work. - Inspect the work area to ensure it is left in a safe and orderly condition. - Lock the backhoe and secure it to prevent unauthorised use. - Report any damage or problems encountered to maintenance or a supervisor. |  |  |  |  |  |  |