| TASK | Band Saw |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☒ | Heat / cold | ☐ | Electricity | ☒ |
|  | Cuts / laceration | ☒ | Dust | ☒ | Rollover | ☐ |
|  | Pinch / crush | ☐ | Noise / vibration | ☒ | Plant interaction | ☐ |
|  | High pressure | ☐ | Other: | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Conduct a thorough inspection of the band saw for any signs of damage or wear, particularly on the blade and safety guards. - Ensure the blade is properly tensioned and sharp. - Check that the blade is suitable for the material and type of cut you plan to make. - Verify the band saw table and guide adjustments are secure and properly aligned according to the material thickness and desired cut. - Secure any loose clothing, jewellery or hair. - Clean the work area of any debris or obstructions. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Secure smaller workpieces with clamps or a vice to keep hands at a safe distance. - Keep fingers and hands out of the path of the blade. Use push sticks or push blocks where possible. - Ensure the workpiece is flat against the table before beginning the cut. - Start the band saw and wait for it to reach full speed before feeding the workpiece into the blade. - Feed the material into the blade at a steady, moderate pace. - Maintain a stance that allows for control of the workpiece without overreaching. - Keep the area around the band saw clear to avoid tripping or stumbling while operating the machine. - Never attempt to remove cut-offs or scraps near the blade while the machine is running. Wait until the blade has stopped. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off the band saw and wait for the blade to come to a complete stop before adjusting or cleaning the area. - Disconnect the band saw from the power source before changing the blade or performing any maintenance. - Clean the table, throat plate, and surrounding area of sawdust and debris. - Inspect the blade for signs of wear or damage, replacing it as necessary. - Store in a dry, secure place to protect it from the elements and unauthorized use. - Report any defects or problems noted during use or inspection. - Document the maintenance and usage log, if applicable, for future reference and compliance. |  |  |  |  |  |  |