| TASK | BENCH GRINDER |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☒ | Heat / cold | ☒ | Electricity | ☒ |
|  | Cuts / laceration | ☒ | Dust | ☒ | Rollover | ☐ |
|  | Pinch / crush | ☐ | Noise / vibration | ☒ | Plant interaction | ☐ |
|  | High pressure | ☐ | Other: | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Inspect the grinder for damage or defects, including the power cord and plug. - Check the tool rests to ensure they are adjusted to within 1/8 inch of the grinding wheels. - Verify that grinding wheels are free from cracks and properly secured. - Ensure that eye shields are in place and not damaged. - Confirm that the area around the grinder is clear of flammable materials and debris. - Make sure the grinder is securely mounted to the bench. - Check that the wheels are appropriate for the material being worked and for the speed of the grinder. - Test the emergency stop function, if available. - Secure loose clothing, hair and jewellery before use. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Standing to one side, turn on the grinder and allow it to reach full speed before using. - Reposition to the front of the grinder. Maintain a proper stance: feet shoulder-width apart for stability, avoid reaching too far. - Keep fingers and hands clear of the rotating wheel. - Hold the workpiece firmly while lowering it to the wheel gradually. - Use the tool rest as support during grinding. - Apply steady, light pressure without forcing the workpiece into the wheel. - Do not grind on the side of the wheel, unless it is specifically designed for side grinding. - Never leave the grinder running unattended. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off the grinder and wait for the wheels to come to a complete stop. - Disconnect grinder from the power source. - Clear away any debris and clean the work area. - Check the grinder for any new signs of wear or damage. - Store all tools and accessories in their designated places. - Report any defects or problems noted during use or inspection. - Document the maintenance and usage log, if applicable, for future reference and compliance. |  |  |  |  |  |  |