| TASK | Brake Press |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☒ | Heat / cold | ☐ | Electricity | ☒ |
|  | Cuts / laceration | ☒ | Dust | ☐ | Rollover | ☐ |
|  | Pinch / crush | ☒ | Noise / vibration | ☐ | Plant interaction | ☐ |
|  | High pressure | ☐ | Other: | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Inspect the press brake for any damage or wear. - Check the hydraulic fluid level and look for leaks. - Ensure safety guards and light curtains are functioning properly. - Verify that the tooling (punch and die) is correctly aligned and secure. - Confirm material thickness and type match the machine settings. - Secure any loose clothing, hair or jewellery. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Use both hands to operate the machine controls when making a bend. - Keep hands and fingers away from the die and punch areas during operation. - Do not place your hand between the material and the die or punch. - Operate the press brake at designated speeds without exceeding recommended limits. - Never attempt to bypass safety controls or interlocks. - Keep the work area clean and free of obstacles. - Do not adjust the machine or change tooling while the machine is operating. - Use proper lifting techniques for heavy materials. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off and disconnect the press brake when finished. - Remove any scrap materials or workpieces from the machine. - Clean the press brake and surrounding area of any debris. - Inspect the machine for any new signs of wear or damage. - Store in a dry, secure place to protect it from the elements and unauthorized use. - Report any defects or problems noted during use or inspection. - Document the maintenance and usage log, if applicable, for future reference and compliance. |  |  |  |  |  |  |