| TASK | Concrete Truck |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☒ | Dust | ☐ | Rollover | ☒ |
|  | Pinch / crush | ☒ | Noise / vibration | ☒ | Plant interaction | ☒ |
|  | High pressure | ☐ | Other: | ☐ | Chemical | ☒ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Inspect the exterior for any signs of concrete buildup, damage, or wear on the barrel and chute. - Complete all pre-start checks as per manufacturer’s guidelines. - Inspect the water tank level to ensure there's enough for the day’s operations. - Inspect hydraulics to ensure no leaks/ damage, and the drum turns correctly. - Confirm that the mixer controls and emergency stop feature are operational. - Adjust the driver’s seat, mirrors and steering wheel where able and required. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Ensure personnel are competent and authorised. - Ensure personnel are fit for duty and not impaired by drugs or alcohol. - Do not use a mobile phone while operating. - Always wear your seatbelt. - Start the truck and let the engine warm up, watching the gauges or indicators for any abnormalities. - Rotate the concrete mixer drum and ensure it’s turning properly. - Operate the vehicle according to road regulations, being mindful of the truck's stability due to the dynamic load. - Maintain communication with the job site to coordinate the arrival. - While on-site, operate at a safe speed and maintain exclusion zones around people, powerlines, and other hazards (use spotters if necessary). - Follow proper procedure for positioning and stabilising the truck before discharging the concrete. - Utilise the drum controls to discharge concrete at the required pace, making sure the chute is directed correctly and securely. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Clean the drum and chute thoroughly. - Conduct a post-operation walk-around to check for any new damage, wear or concrete that may have been missed during cleaning. - Record and document any irregularities or issues encountered during operation. - Park the truck in the designated area, ensuring the drum is empty and clean, ready for the next batch. - Remove the ignition keys and secure the vehicle, making sure all compartments and storage boxes are locked. Return keys and update logbook as required. |  |  |  |  |  |  |