| TASK | Concrete Vibrator |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☒ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☐ |
|  | Pinch / crush | ☒ | Noise / vibration | ☒ | Plant interaction | ☐ |
|  | High pressure | ☐ | Strain | ☒ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Ensure all personnel are trained and authorized to operate the concrete vibrator. - Inspect the vibrator for any visible damage, loose components, or signs of wear. - Check that the vibrator shaft is securely attached to the motor and that the shaft length is appropriate for the job. - Verify that the power cord is in good condition and that the power switch is functioning correctly. - Confirm that fire extinguishing equipment is readily available in case of emergency. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Wear appropriate personal protective equipment (PPE) including safety glasses, hearing protection, gloves, and steel-toed boots. - Position the vibrator shaft into the fresh concrete mixture, ensuring that it penetrates evenly and does not contact any reinforcement. - Turn on the vibrator motor and gradually immerse the shaft into the concrete, avoiding rapid or jerky movements. - Move the vibrator shaft through the concrete in a systematic pattern, ensuring thorough consolidation of the mixture. - Be aware of any obstacles or obstructions in the concrete that may impede the vibrator's movement. - Keep a firm grip on the vibrator handle and maintain control of the equipment at all times. - If any issues or errors occur during operation, stop the vibrator immediately and troubleshoot the problem before resuming. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off the vibrator motor and remove the shaft from the concrete, ensuring that it is clean and free of any hardened material. - Inspect the concrete surface for any defects or areas that may require additional consolidation. - Clean the vibrator shaft and motor thoroughly, removing any concrete residue or debris. - Store the vibrator and associated equipment in a designated area, ensuring they are protected from damage and unauthorized use. |  |  |  |  |  |  |