| TASK | Demolition Saw |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☒ | Heat / cold | ☒ | Electricity | ☒ |
|  | Cuts / laceration | ☒ | Dust | ☒ | Rollover | ☐ |
|  | Pinch / crush | ☒ | Noise / vibration | ☒ | Plant interaction | ☐ |
|  | High pressure | ☐ | Other: | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Check for visible defects or damage. - Check cord for visible defects or damage and tested-and-tagged (for 240v). - Check fuel and fluid levels and no leaks (for petrol). - Check guards are in place. - Ensure appropriate PPE is worn. - Tie back loose clothing, jewellery or hair. - Ensure other workers are not in the area. - Ensure dust controls are implemented. - Ensure flammables are removed. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Hold the saw with both hands, one hand on the handle and the other on the auxiliary handle. - Maintain a stable stance, positioning your body to the side of the saw and away from the cutting path. - Start the saw following the manufacturer's instructions and allow it to reach full speed before engaging with the material. - Approach the material with a slow, controlled motion, allowing the blade to cut steadily. - Avoid forcing the saw through the material; let the blade do the work. - Maintain a firm grip on the saw handles and be aware of the blade's position at all times. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off the demolition saw and allow the blade to come to a complete stop before setting the tool down. - For petrol-powered saws, turn off the fuel supply before shutting down the engine. - Inspect the saw for any damage or wear, paying particular attention to the cutting blade. - After the saw has cooled, clean the saw, removing any debris or residue from the blade, guards, and handles. - Securely store the demolition saw in a dry and secure location, away from direct sunlight and moisture. |  |  |  |  |  |  |