| TASK | Dowelling Machine |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☒ | Dust | ☒ | Rollover | ☐ |
|  | Pinch / crush | ☒ | Noise / vibration | ☒ | Plant interaction | ☐ |
|  | High pressure | ☐ | Kickback | ☒ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Ensure all personnel are trained and authorized to operate the dowelling machine. - Inspect the dowelling machine for any visible damage, loose components, or signs of wear. - Check that the cutting blades or drill bits are sharp, properly aligned, and secure. - Verify that the workpiece is securely clamped in place and that the machine bed is clear of any obstructions. - Ensure that the power cord is in good condition and that the power switch is functioning correctly. - Confirm that fire extinguishing equipment is readily available in case of emergency. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Wear appropriate personal protective equipment (PPE) including safety glasses, hearing protection, and gloves. - Position the workpiece on the dowelling machine, ensuring that it is properly aligned with the cutting blades or drill bits. - Adjust the depth and spacing of the dowel holes as necessary for the desired joint. - Start the dowelling machine and allow it to reach full speed before cutting - Feed the workpiece into the machine with a steady, controlled motion, keeping fingers away from the cutting area. - Use push sticks or other tools to guide the workpiece through the machine, especially when close to the cutting area. - Be aware of any unusual noises or vibrations during operation and stop the machine immediately if necessary. - If any issues or errors occur during operation, stop the dowelling machine immediately and troubleshoot the problem before resuming. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off the dowelling machine and wait for all moving parts to come to a complete stop before removing the workpiece. - Inspect the dowel holes and the workpiece for any defects or issues, reporting any problems to the appropriate personnel. - Clean up the work area, removing any wood chips or debris from the dowelling machine and surrounding area. - Store the dowelling machine accessories and tools in a designated area, ensuring they are protected from damage and unauthorized use. |  |  |  |  |  |  |