| TASK | Drill Press |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☒ | Heat / cold | ☒ | Electricity | ☒ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☐ |
|  | Pinch / crush | ☒ | Noise / vibration | ☒ | Plant interaction | ☐ |
|  | High pressure | ☐ | Other: | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Inspect the drill press for any signs of wear, damage, or loose components. - Ensure that the chuck key is removed before operation. - Check the drill bit for sharpness and damage. Replace if necessary. - Confirm that the worktable is clean and free of debris. - Verify that the workpiece is securely clamped or vice-held to the table. - Adjust the table or spindle to the correct height and ensure it's locked in place. - Check that the correct spindle speed is set for the material and drill bit size. - Ensure all guards are in place and functional. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Use only the designated start controls to activate the drill press. - Keep hands and fingers away from the rotating drill bit. - Do not perform layout or measurement tasks on the workpiece while the machine is running. - Apply cutting fluid if required for the material being drilled. - Feed the drill bit into the material with steady, even pressure – do not force it. - If the drill bit binds in the workpiece, turn off the machine immediately. Do not attempt to free the bit while the machine is powered. - Keep the floor around the drill press clean and dry to prevent slipping. - Clean the drill bit and worktable of any chips or debris using a brush or vacuum – never with your hands. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off the drill press and wait for the drill bit to come to a complete stop. - Unclamp and remove the workpiece from the table. - Clean the drill bit (once cooled) and worktable of any chips or debris using a brush or vacuum. - Inspect the drill bit for wear or damage and store appropriately. - Disconnect the drill press from power if it will not be used for an extended period. - Report any defects or problems noted during use or inspection. - Document the maintenance and usage log, if applicable, for future reference and compliance. |  |  |  |  |  |  |