| TASK | Drill |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☒ | Heat / cold | ☒ | Electricity | ☒ |
|  | Cuts / laceration | ☒ | Dust | ☒ | Rollover | ☐ |
|  | Pinch / crush | ☐ | Noise / vibration | ☒ | Plant interaction | ☐ |
|  | High pressure | ☐ | Other: | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Ensure the drill is clean and free of any damage. - Check for worn or damaged parts. - Ensure the work area is clean and free of any hazards. - Confirm adequate lighting is available. - Select the correct drill bit for your material and task. - Inspect drill bits for sharpness and damage. - Ensure you have the appropriate personal protective equipment (PPE). - Tie back loose clothing, jewellery or hair. - Use clamps or a vice to securely hold the material in place. - Ensure the drill is properly and safely plugged in or that the battery is fully charged for cordless drills. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Stand in a stable position where you have control of the drill and are not overreaching. - Turn on the drill only when you are prepared to begin your work. - If possible, use both hands to operate the drill, keeping one on the handle and the other supporting the drill's body for stability. - Apply steady pressure when drilling; avoid forcing the drill too hard or too fast into the material. - Adjust the speed according to the material being drilled and the diameter of the hole. - Slow down as you approach the end of the drilling process to avoid damage or injury when the bit breaks through the other side. - Allow the bit to cool down before touching it after use, as it can become very hot. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off and unplug the drill (or remove the battery if cordless) when the task is completed. - Clean the drill and bits of any debris and wipe down the equipment. - After cooling, inspect the drill and bits for wear or damage. - Properly store the drill, bits, and any other accessories in a dry, secure location. - Report any malfunction or damage to responsible party for maintenance or repair. - If required, document the completion of the task and any issues encountered for future reference. |  |  |  |  |  |  |