| TASK | Extension Ladders |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☒ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☐ |
|  | Pinch / crush | ☐ | Noise / vibration | ☐ | Plant interaction | ☐ |
|  | High pressure | ☒ | Falling | ☒ | Trips | ☒ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Inspect the extension ladder for any visible damage, cracks, or defects. - Ensure the ladder is the appropriate length for the task at hand and can reach the required height. - Check the ladder's weight rating to ensure it safely supports the anticipated load. - Make sure the ladder's feet are in good condition and equipped with non-slip materials or rubber caps. - Choose a stable and level surface on which to place the ladder. - Put on appropriate personal protective equipment (PPE) for ladder usage. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Set up the ladder on a stable surface, ensuring it is positioned at the correct angle following the 4-to-1 rule (1 foot out for every 4 feet of ladder height). Secure the ladder to prevent slippage. - Ascend or descend the ladder facing the ladder and maintaining three points of contact (two hands and one foot or two feet and one hand) at all times. - Always maintain a firm grip on the ladder rungs or side rails during use. - Do not overreach while on the ladder; reposition it instead. - Avoid carrying heavy or bulky items up or down the ladder, using a tool belt or hoisting equipment instead. - Do not exceed the ladder's weight capacity and avoid any sudden movements or shifts that may compromise stability. - Be cautious of electrical hazards and keep the ladder away from exposed wires or power lines to prevent electrical shocks. - Communicate with others in the vicinity when using the ladder and ensure they maintain a safe distance to prevent accidental collisions. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Inspect the ladder for any damage, wear, or loose components. Remove from service if any issues are found. - Clean any debris off the ladder and ensure it is stored properly in a designated area. - Report any ladder-related incidents, near misses, or safety concerns to the appropriate personnel. - Ensure other workers are aware of the ladder's condition and if it needs maintenance or repairs. |  |  |  |  |  |  |