| TASK | Forklift |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☒ |
|  | Pinch / crush | ☒ | Noise / vibration | ☐ | Plant interaction | ☒ |
|  | High pressure | ☐ | Falling / struck-by | ☒ | Chemical | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Examine the entire body of the forklift for any signs of damage or hazards, including dents, cracks, or rust that could affect operation or safety. - Ensure the forks and any attachments are securely attached. - Check for any cracks or bends in the forks. - Inspect fluid levels (hydraulic fluid, coolant, brake fluid, etc.) and top up as needed, while checking for any signs of leaks. - Test the functionality of the hydraulic lift, making sure it operates smoothly and securely. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Observe the speed limits and use caution when turning, especially when carrying a load. - Keep a safe distance from pedestrians, obstacles, and other vehicles. - Use the horn when approaching intersections, blind spots, or when visibility is limited. - Always drive in reverse with a load if the load obstructs forward visibility. - Ensure the load is stable and properly secured on the forks. - Use the appropriate lifting capacity for the load and keep it within the forklift's rated capacity. - Tilt the mast slightly backward while lifting to stabilize the load. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - After use, perform a walk-around check of the forklift to inspect for any new damage incurred during operation. - Check for any damage or leaks that may have occurred during operation. - Refuel or recharge the forklift if necessary. - Report any issues or concerns to the appropriate personnel for maintenance or repair. |  |  |  |  |  |  |