| TASK | Front-End Loader |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☒ |
|  | Pinch / crush | ☒ | Noise / vibration | ☒ | Plant interaction | ☒ |
|  | High pressure | ☐ | Other: | ☐ | Chemical | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Walk around to inspect for visible damage, wear, or leaks. - Complete all pre-start checks as per manufacturer’s guidelines. - Inspect attachments for wear or breakage. - Test all control functions in the operator's cabin to verify they are working correctly. - Test all lights, beacons, and signalling devices to ensure operational. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Ensure personnel are competent and authorised. - Ensure personnel are fit for duty and not impaired by drugs or alcohol. - Adjust the operator’s seat and mirrors for maximum visibility - Do not use a mobile phone while operating. - Always wear a seat belt. - Start the engine and observe the dashboard for any warning lights or abnormal gauge readings. - Test all loader controls, including lift, tilt, and articulation, for smooth operation. - Operate the loader at a moderate pace to maintain stability, especially when carrying loads. - Use the loader's lift and tilt functions to load and unload materials efficiently and safely. - Operate at a safe speed and maintain exclusion zones around people, powerlines, and other hazards (use spotters if necessary). - Practice smooth driving techniques to protect the equipment and load and minimise spillage. - Bewar of uneven or sloping ground. - Never carry people or lift loads over people. - Use only suitable lifting gear and attachments. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Clean any remaining materials from the bucket and chassis. - Lower the bucket to the ground and place the loader in a stable, park position. - Review the loader for any new signs of wear or damage. - Turn off the engine, lock the vehicle (if applicable) and securely store the key. - Record the operating hours and note any issues in the equipment log. - Report any problems or maintenance requirements to supervisors or maintenance staff. |  |  |  |  |  |  |