| TASK | Grader |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☒ |
|  | Pinch / crush | ☒ | Noise / vibration | ☒ | Plant interaction | ☒ |
|  | High pressure | ☐ | Other: | ☐ | Chemical | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Walk around to inspect for visible damage, wear, or leaks. - Complete all pre-start checks as per manufacturer’s guidelines. - Test all control functions in the operator's cabin to verify they are working correctly - Inspect the cabin's controls, seatbelt, and operator seat for safety and comfort. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Ensure personnel are competent and authorised. - Ensure personnel are fit for duty and not impaired by drugs or alcohol. - Do not use a mobile phone while operating. - Always wear a seatbelt. - Start the engine, allowing it to warm up while monitoring gauges for anything abnormal. - Test all control functions including blade lift, tilt, angle, and side shift. - Begin operation with attention to blade position to achieve the desired surface finish. - Operate at a safe speed and maintain exclusion zones around people, powerlines, and other hazards (use spotters if necessary). - Beware of uneven or sloping ground. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Clean the blade and scarifier of any debris or buildup. - Position the grader on level ground and lower the blade and scarifier to the ground. - Thoroughly inspect the equipment for any signs of new wear or damage. - Record the machine’s hours and any functional issues encountered. - Turn off the engine, remove the keys, and perform lock-out procedures to secure the machine. - Complete any required reports or logs detailing the day’s operations and machine status. - Notify the maintenance team of any repairs or servicing needed. |  |  |  |  |  |  |