| TASK | Hammer Drill |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☒ | Heat / cold | ☒ | Electricity | ☒ |
|  | Cuts / laceration | ☒ | Dust | ☒ | Rollover | ☐ |
|  | Pinch / crush | ☐ | Noise / vibration | ☒ | Plant interaction | ☐ |
|  | High pressure | ☐ | Other: | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Examine the tool for any visible damage or defects. - Ensure the cord (if applicable) and plug are in good condition. For battery-operated models, check that the battery is charged. - Verify that the chuck and any safety guards are tight and properly positioned. - Choose a drill bit that is appropriate for the material and size of the hole. - Check the condition of the drill bit, ensuring it is sharp and undamaged. - Clamp down or securely support the material being drilled to prevent movement. - Ensure a clean, debris free, dry, and well-lit work area. - Tie back loose clothing, jewellery or hair. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Firmly hold the hammer drill with both hands (when possible) to maintain control. - Plug in the drill (for corded models) or ensure the battery is securely attached (for cordless models). - Turn on the drill and allow it to reach full speed before drilling. - Position the drill bit at the desired drilling point. - Apply steady pressure in a straight line with the bit. Do not force the drill into the material; let the drill do the work. - Ensure you know the depth you need to drill to avoid drilling too deeply. Consider using a depth stop if available. - Adjust the drill’s speed and torque settings according to the hardness of the material and the size of the drill bit. - Periodically withdraw the drill bit from the hole to clear debris and allow the bit to cool down. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off and unplug the hammer drill (or remove the battery if cordless). - Wait for the drill bit to cool down before removing it from the chuck. Store the bit properly. - Wipe down the hammer drill, removing any debris and dust from the tool. - Conduct an inspection for any damage that might have occurred during use. - Report any malfunction or damage to responsible party for maintenance or repair. - If required, document the completion of the task and any issues encountered for future reference. |  |  |  |  |  |  |