| TASK | Laser Cutter |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☒ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☐ |
|  | Pinch / crush | ☒ | Noise / vibration | ☐ | Plant interaction | ☐ |
|  | High pressure | ☐ | Chemical | ☒ | Eye Irritation | ☒ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Ensure all personnel are trained and authorized to operate the laser cutter. - Inspect the laser cutter for any visible damage or signs of wear. - Check that the laser tube, focusing lens, and mirrors are clean and aligned. - Verify that the work area is clear of flammable materials and properly ventilated. - Ensure it is connected to a suitable power source with all all connections secure. - Confirm that fire extinguishing equipment is readily available in case of emergency. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Wear appropriate personal protective equipment (PPE) including safety glasses with side shields, a face shield, and gloves. - Secure the workpiece using clamps or fixtures to prevent movement during cutting. - Set up the laser cutter parameters such as power, speed, and frequency according to the material type and thickness. - Load the file into the laser cutter software and verify the settings before initiating. - Activate the laser cutter and initiate the cutting or engraving process using the control panel or software interface. - Monitor the operation closely, ensuring that the laser beam remains focused on the workpiece and that cutting or engraving progresses smoothly. - Be aware of any smoke, fumes, or odors generated during cutting or engraving and ensure proper ventilation is maintained. - Avoid opening the enclosure or interfering with the laser beam while in operation. - After cutting or engraving is complete, turn off the laser cutter and allow it to cool down before opening the enclosure or removing the workpiece |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Inspect the cut or engraved workpiece for quality and completeness, making any necessary adjustments to the cutting parameters for future reference. - Clean up the work area, removing any debris or waste material generated during cutting or engraving. - Dispose of scrap material and any waste materials such as used protective film or masking tape properly according to company guidelines. - Perform routine maintenance tasks such as cleaning the lens and mirrors, checking for loose connections, and inspecting the ventilation system. - Store the laser cutter and accessories in a designated area, ensuring they are protected from damage and unauthorized use. |  |  |  |  |  |  |