| TASK | HC / MC Truck |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☒ |
|  | Pinch / crush | ☒ | Noise / vibration | ☐ | Plant interaction | ☒ |
|  | High pressure | ☐ | Other: | ☐ | Chemical | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Walk around the truck, inspecting for any visible damage to the body or structure. - Complete all pre-start checks as per manufacturer’s specifications. - Ensure that coupling devices (fifth wheel, kingpin, airlines) are secure, intact, and lubricated. - Confirm that required safety equipment, like fire extinguishers and reflective triangles, is present and in good condition. - Adjust the seat and adjust mirrors from inside the cab for optimal driving posture and visibility. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Ensure personnel are competent and authorised. - Ensure personnel are fit for duty and not impaired by drugs or alcohol. - Do not use a mobile phone while operating. - Always wear your seat belt. - Start the engine and let it idle, watching for warning lights. - Conduct a brake check to ensure proper function. - Keep to road rules, use suitable gears for your load, and increase your following distance due to the vehicle's size and weight. - Use engine braking judiciously on downhills to maintain control. - Secure the load to prevent shifting that can impact truck stability. - While onsite, operate at a safe speed and maintain exclusion zones around people, powerlines, and other hazards (use spotters if necessary). - Beware of uneven or sloping ground. - Employ the help of a spotter for safe reversing and take care during manoeuvres to avoid rollover or jackknife incidents. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Set the parking brake, turn the ignition off, and remove the key. - After parking, inspect the truck for any new damage or malfunctions that may have occurred during the trip. - Record any mechanical or performance issues in the maintenance log. - Clean the cabin, removing rubbish and personal belongings to leave it tidy. - Log trip details including mileage and fuel usage in the appropriate logbook. - Lock the cab, secure any opened compartments, ensure the vehicle is parked safely in an authorised area and return the keys to their designated area. |  |  |  |  |  |  |