| TASK | Mig Welder |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☒ | Electricity | ☒ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☐ |
|  | Pinch / crush | ☒ | Noise / vibration | ☐ | Plant interaction | ☐ |
|  | High pressure | ☐ | Arc Radiation | ☒ | Fume / Gas | ☒ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Ensure all personnel are trained and authorized to operate equipment. - Inspect the welding machine for any visible damage, loose connections, or wear. - Check that the gas cylinder is securely fastened and contains the appropriate shielding gas for the welding process. - Verify that the welding wire is the correct type and diameter for the material used. - Confirm that fire extinguishing equipment (e.g., fire extinguisher, fire blanket) is readily available in case of emergency. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Wear appropriate personal protective equipment (welding helmet with proper shade lens, safety glasses, welding gloves, and flame-resistant clothing). - Turn on the welding machine and adjust the settings for the desired welding process, voltage, and wire feed speed. - Position the workpiece in a secure and stable position, ensuring proper grounding of the workpiece and welding machine. - Hold the MIG gun at the appropriate angle and distance from the workpiece, maintaining a consistent travel speed. - Begin welding by pulling the trigger on the MIG gun to start the arc, and move steadily along the joint in a straight line or weaving motion as needed. - Ensure proper gas coverage and penetration by adjusting the angle and distance of the MIG gun from the workpiece. - Be aware of any signs of improper welding such as excessive spatter, incomplete fusion, or porosity, and adjust settings or techniques accordingly. - After completing the weld, release the trigger on the MIG gun to stop the arc, and allow the weld to cool before handling or inspecting. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off the machine and allow it to cool before doing maintenance or adjustments. - Inspect the weld for any defects such as cracks, undercut, or incomplete fusion, and make note of any issues for repair or rework. - Clean the work area to remove any spatter, debris, or flux residue from the weld. - Properly store welding equipment and accessories in a designated area, ensuring they are protected from damage and unauthorized use. - Securely fasten the gas cylinder valve and return unused welding wire to its storage spool. |  |  |  |  |  |  |