| TASK | Oxy Acetylene |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☐ |
|  | Pinch / crush | ☐ | Noise / vibration | ☐ | Plant interaction | ☐ |
|  | High pressure | ☒ | Fire / Explosion | ☒ | Flashback | ☒ |
|  | Fumes | ☒ | Burns | ☒ | Eye damage | ☒ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Ensure all personnel are trained and authorized to operate the equipment. - Inspect the cylinders for any visible damage, leaks, or signs of wear. - Check that the regulators, hoses, and fittings are connected and in good condition. - Verify that flashback arrestors are installed on the oxygen and acetylene lines. - Confirm that fire extinguishing equipment (e.g., fire extinguisher, fire blanket) is readily available in case of emergency. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Wear appropriate personal protective equipment (PPE) including safety glasses, welding helmet, gloves, and flame-resistant clothing. - Open the oxygen cylinder valve slowly and fully, then adjust regulator as advised. - Repeat the same process for the acetylene cylinder, ensuring the pressure does not exceed the recommended limit. - Light the acetylene torch using a friction lighter, keeping the torch pointed away from yourself and others. - Adjust the acetylene flow to produce a neutral flame by gradually opening the oxygen valve until the flame stops smoking and becomes sharp. - Position the torch tip approximately 1/8 to 1/4 inch from the workpiece, and adjust the flame as necessary for cutting or welding. - Preheat the metal until its cherry red before applying the oxygen jet to initiate a cut. - During welding, maintain the torch angle and distance to achieve proper penetration and fusion. - Be vigilant for any signs of flashback or backfire, such as popping or hissing noises, and shut off the torch immediately if detected. - After use, close the acetylene valve first, followed by the oxygen valve, and bleed off any remaining gas from the hoses. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Shut off the regulators and bleed the hoses to release any remaining gas pressure. - Inspect the equipment for any damage or leaks, and report any issues. - Store cylinders in an upright position, secured to prevent tipping or damage. - Clean the torch tip and hoses to remove any debris or contaminants. - Ensure all equipment is properly stored in a designated area, and the work area is clear of any flammable materials. |  |  |  |  |  |  |