| TASK | Pallet Jack |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☐ |
|  | Pinch / crush | ☒ | Noise / vibration | ☐ | Plant interaction | ☐ |
|  | High pressure | ☐ | Kickback | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Inspect the pallet jack for any physical damage, especially on the forks and the hydraulic pump. - Check the wheels to ensure they are intact, secure and roll freely without any obstructions. - Examine the handle for secure attachment and test the control lever for lifting, lowering and neutral operation. - Look over the hydraulic oil reservoir to ensure it's not leaking and has sufficient oil. - Confirm there is no debris in the path of travel and that the work area is clear. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Place the pallet jack's forks fully into the pallet, ensuring they're centred, and the pallet is balanced. - Pump the handle to raise the pallet off the ground, checking that the lift mechanism operates smoothly and the pallet is stable. - Push the pallet jack handle forward to the neutral position before moving. - Pull or push the pallet jack at a controlled pace, keeping the path ahead in clear view and avoiding sudden stops or sharp turns. - Use the control lever to carefully lower the pallet once destination is reached and before removing the forks from the pallet. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Upon completion of work, ensure the forks of the pallet jack are fully lowered to the ground. - Conduct a final inspection of the pallet jack, looking for any damage or excessive wear incurred during operations. - Clean the forks and wheels of the pallet jack to remove any debris that could impede function. - Store the pallet jack in its designated area so it does not hinder traffic or pose a safety hazard. - Report any operational issues or defects noticed during the post-use inspection to the supervisor or maintenance team for further action. |  |  |  |  |  |  |