| TASK | Pallet Stacker |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☒ | Dust | ☐ | Rollover | ☒ |
|  | Pinch / crush | ☒ | Noise / vibration | ☐ | Plant interaction | ☐ |
|  | High pressure | ☐ | Kickback | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Examine the pallet stacker for any visible signs of damage, including the forks, mast, and body of the machine. - Ensure that the battery is fully charged or sufficiently fuelled and check for any signs of leaks or battery corrosion. - Verify that all control buttons and levers are clearly marked and fully functional. - Test the brakes, horn, and emergency stop feature to confirm responsiveness. - Look over the wheels and casters for wear or damage and ensure they rotate freely and are not obstructed by debris. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Position yourself correctly within the operator’s area and ensure a firm footing and good posture if the stacker is a walk-behind model. - Use the control handle to carefully raise and lower the forks to ensure they are functioning as expected before loading. - Gently throttle the stacker to confirm smooth operation and manoeuvrability. - Approach the load squarely and carefully slide the forks fully under the pallet, making sure the pallet is centred and secure. - Lift the pallet just enough to clear the floor for transport, watching closely for overhead obstacles when stacking or tiering. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Lower any loads to the ground and return the forks to the minimum height position before switching off the pallet stacker. - Conduct a final inspection for any new signs of damage or wear after operation. - Clean the equipment with attention to removing any debris from the working mechanisms or wheels. - Recharge the battery or refuel if necessary, following manufacturer’s instructions or company protocols. - Record the usage in a logbook and report any maintenance concerns to ensure the stacker is ready and safe for the next use. |  |  |  |  |  |  |