| TASK | Plasma Cutter |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☒ |
|  | Cuts / laceration | ☐ | Inhalation | ☒ | Rollover | ☐ |
|  | Pinch / crush | ☐ | Noise / vibration | ☒ | Plant interaction | ☐ |
|  | High pressure | ☐ | Fire / explosion | ☒ | Arc Flash | ☒ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Ensure all personnel are trained and authorized to operate the plasma cutter. - Inspect the plasma cutter for any visible damage or signs of wear. - Check the cutting torch, consumables, and grounding clamp are properly installed and in good condition. - Verify that the work area is clear of flammable materials and properly ventilated. - Ensure the machine is connected to a suitable power source with secure connections. - Confirm that fire extinguishing equipment is readily available in case of emergency. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Wear appropriate personal protective equipment (including safety glasses with side shields, a welding helmet with shaded lens, flame-resistant clothing, and gloves). - Secure the workpiece is placed using clamps or fixtures to prevent movement. - Adjust the plasma cutter settings according to the material thickness and type. - Hold the cutting torch at the correct angle and distance from the workpiece to achieve clean and precise cuts. - Activate the cutter and initiate cutting by contacting the torch to a workpiece. - Move the torch along the cutting path smoothly and steadily, maintaining a consistent speed to prevent overheating or distortion of the material. - Be aware of sparks, slag, and hot metal debris generated during cutting and ensure they do not pose a hazard to nearby personnel or equipment. - Monitor the cutting progress and adjust the torch position or settings as necessary to achieve the desired results. - After cutting is complete, turn off the plasma cutter and allow it to cool down before performing any maintenance or adjustments. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Inspect the cut edges for smoothness and quality, making any necessary adjustments to the cutting parameters for future reference. - Clean up the work area, removing any debris or waste material generated. - Dispose of scrap material and consumables properly according to company policy. - Store the plasma cutter and accessories in a designated area, ensuring they are protected from damage and unauthorized use. - Perform routine maintenance tasks such as cleaning the torch, replacing consumables, and inspecting electrical connections to ensure safe operation. |  |  |  |  |  |  |