| TASK | Pressure Washer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☒ | Heat / cold | ☒ | Electricity | ☒ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☐ |
|  | Pinch / crush | ☐ | Noise / vibration | ☒ | Plant interaction | ☐ |
|  | High pressure | ☒ | Other: | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Inspect the pressure washer for any damage, leaks, or worn parts. - Check hoses and connections to ensure they're secure and not damaged. - Confirm that the cleaning nozzle is appropriate for the task. - Clear the area of debris and secure loose objects that might become projectiles. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Connect the pressure washer to the water supply and turn it on according to the manufacturer's instructions. - Start with the nozzle held at a safe distance from the surface to be cleaned, gradually moving closer as needed. - Maintain a firm grip on the spray gun. The force of the water can make it difficult to control. - Avoid directly spraying windows, electrical outlets, or any electrical appliances to prevent damage and electrical hazards. - Never point the pressure washer at people, animals, or plants. - Use the pressure washer in an open, well-ventilated space to minimize the risk of inhaling fumes from gas-powered models. - Be mindful of slippery surfaces when working with water. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off the pressure washer and disconnect it from the water supply. - Release any remaining pressure carefully according to the manufacturer's instructions. - Clean and inspect the nozzle and spray gun for blockages or damage. - Drain hoses and the pressure washer. - Store the pressure washer in a dry, safe place out of the reach of children. - Report any defects or problems noted during use or inspection. - Document the maintenance and usage log, if applicable, for future reference and compliance. |  |  |  |  |  |  |