| TASK | Rattle Gun |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☒ | Heat / cold | ☐ | Electricity | ☒ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☐ |
|  | Pinch / crush | ☐ | Noise / vibration | ☒ | Plant interaction | ☐ |
|  | High pressure | ☒ | Other: | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Inspect the rattle gun for visible damage. - Ensure the air hose or power cord is in good condition, with no cuts or abrasions. - Check the air compressor (for pneumatic models) is operating at the correct pressure. - Verify the battery is fully charged for cordless models. - Select the correct socket size for the task. - Attach the socket securely to the rattle gun. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Connect the rattle gun to the air supply or power source. - Hold the rattle gun firmly with both hands when in use. - Position yourself to avoid kickback and maintain balance. - Place the socket onto the fastener and activate the rattle gun smoothly. - Apply continuous pressure until the fastening is complete or the fastener is removed. - Disconnect the rattle gun from the workpiece before turning it off. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Disconnect the rattle gun from the power source or air supply. - Remove the socket from the rattle gun and clean any debris from the tool. - Inspect the tool, hose, and connectors for damage. - Store the rattle gun, sockets, and accessories in a clean, dry place. - Report any defects or problems noted during use or inspection. - Document the maintenance and usage log, if applicable, for future reference and compliance. |  |  |  |  |  |  |