| TASK | Reciprocating Saw |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☒ | Heat / cold | ☐ | Electricity | ☒ |
|  | Cuts / laceration | ☒ | Dust | ☒ | Rollover | ☐ |
|  | Pinch / crush | ☐ | Noise / vibration | ☒ | Plant interaction | ☐ |
|  | High pressure | ☐ | Other: | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Inspect the reciprocating saw for any visible damage or defects. - Check the condition and sharpness of the blade; replace if worn or damaged. - Ensure the blade is securely fastened in the saw. - Verify the battery is fully charged for cordless models or that the power cord is undamaged for corded models. - Confirm the work area is secure, well-lit, and free of hazards. - Tie back loose clothing or hair. - Clamp or securely support the material to be cut. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Hold the saw firmly with both hands, maintaining a stable and balanced stance. - Ensure blade is not in contact with the material before turning on the saw. - Turn on the saw and allow it to reach full speed before beginning the cut. - Use steady, controlled pressure to guide the saw through the material. - Be aware of the potential for kickback and maintain a firm grip. - Do not twist the blade while cutting. - Keep the cord or battery pack away from the cutting path. - Turn off the saw and wait for the blade to stop completely before pulling it out of the cut. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off the saw and disconnect the power source. - Wait for the blade to stop moving before setting the saw down. - Remove the blade (once cooled) if it is damaged or if the saw will be stored for a long period. - Clean the saw body, blade, and ventilation slots of any debris or damage. - Check the saw and blade for signs of wear or damage. - Report any defects or problems noted during use or inspection. - Document the maintenance and usage log if applicable. |  |  |  |  |  |  |