| TASK | Roller |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☒ |
|  | Pinch / crush | ☒ | Noise / vibration | ☒ | Plant interaction | ☒ |
|  | High pressure | ☐ | Other: | ☐ | Chemical | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Walk around to inspect for visible damage, wear, or leaks. - Complete all pre-start checks as per manufacturer’s guidelines. - Test all control functions in the operator's cabin to verify they are working correctly. - Verify that the seatbelt works, and the seat is adjustable and secure. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Ensure personnel are competent and authorised. - Ensure personnel are fit for duty and not impaired by drugs or alcohol. - Do not use a mobile phone while operating. - Always wear a seatbelt. - Start the engine, allowing it to warm up while monitoring gauges for anything abnormal. - Conduct a control check to ensure all systems, including brakes and vibration, are responsive. - Operate at a safe speed and maintain exclusion zones around people, powerlines, and other hazards (use spotters if necessary). - Use the vibration feature only when necessary to avoid damage to the machine or work surface. - Maintain steady, overlapping passes for uniform compaction. - Beware of uneven or sloping ground. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Clean the drums to remove any stuck material that could harden. - Position the roller in a safe, flat area away from operational traffic. - Lower any ROPS (Roll Over Protection System) bars if they're in an elevated position. - Turn off the machine and remove the keys to prevent unauthorised usage. - Record the machine’s hour meter reading along with any maintenance observations. - Secure the roller by locking cabins or hatches if available. - Report any damage or issues found during operation to the appropriate supervisor or maintenance staff. |  |  |  |  |  |  |