| TASK | Scissor Lifts |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☒ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☒ |
|  | Pinch / crush | ☒ | Noise / vibration | ☐ | Plant interaction | ☒ |
|  | High pressure | ☐ | Falling hazard | ☒ | Tip over | ☒ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Inspect the scissor lift for any visible damage, leaks, or defects. - Conduct pre-start check as per manufacturer’s guidelines. - Check the stability of the ground and ensure it can support the weight of the scissor lift. - Verify that all safety features, such as guardrails and emergency stop buttons, are functional. - Confirm that all controls and systems are operational by performing a test lift and lowering. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Ensure personnel are competent and authorised. - Ensure personnel are fit for duty and not impaired by drugs or alcohol. - Do not use a mobile phone while operating. - Maintain exclusion zones around people, powerlines, and other hazards (use spotters if necessary). - Beware of uneven or sloping ground. - Use the provided harness and attach to the designated anchor point before ascending. - Operate the scissor lift smoothly, avoiding sudden movements or overloading. - Lower the platform completely and secure it before exiting the scissor lift. - Never carry people or lift loads over people. - Use only suitable gear and attachments. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Lower the platform to the ground and turn off the scissor lift. - Perform a final visual inspection for any damage or debris. - Secure the scissor lift and return it to the designated storage area. - Report any issues or incidents encountered during operation to the appropriate personnel. |  |  |  |  |  |  |