| TASK | Scraper |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☒ |
|  | Pinch / crush | ☒ | Noise / vibration | ☒ | Plant interaction | ☒ |
|  | High pressure | ☐ | Other: | ☐ | Chemical | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Examine the scraper bowl, cutting edges, and apron for structural damage or excessive wear. - Conduct all pre-start checks as per manufacturer’s guidelines. - Confirm all controls are operational and that the seat belt is in good condition. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Ensure personnel are competent and authorized. - Ensure personnel are fit for duty and not impaired by drugs or alcohol. - Adjust the operator seat for visibility and control access; fasten the seatbelt. - Do not use a mobile phone while operating and always wear your seatbelt. - Start the engine, allow it to warm up. Monitor for unusual noises or warning lights. - Test the functionality of the scraper’s bowl lift, ejection, and steering controls. - Operate at a safe speed and maintain exclusion zones around people, powerlines, and other hazards (use spotters if necessary). - Beware of uneven or sloping ground. - Cut and load material at an appropriate, consistent depth, and adjust speed according to site conditions. - Transport material to the designated area, adhering to safe driving practices. - Unload material effectively using the scraper’s ejection system, spreading the load evenly. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Remove any material buildup from the scraper’s bowl and ejection area. - Conduct a visual inspection for new signs of damage or wear. - Park the scraper on level ground, lower the bowl to its parking position, and engage the parking brake. - Record the operational hours and any maintenance observations in the equipment log. - Lock the cab and secure the machine against unauthorised use. - Communicate any mechanical issues or damage to maintenance personnel. |  |  |  |  |  |  |