| TASK | Skid-steer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☒ |
|  | Pinch / crush | ☒ | Noise / vibration | ☒ | Plant interaction | ☒ |
|  | High pressure | ☐ | Other: | ☐ | Chemical | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Walk around skidsteer and inspect for wear, tear, or maintenance concerns. - Complete all pre-start check as per manufacturer’s guidelines. - Ensure cabin controls, seat adjustment, and restraint systems are fully operational. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Ensure personnel are competent and authorised. - Ensure personnel are fit for duty and not impaired by drugs or alcohol. - Do not use a mobile phone while operating. - Always wear your seatbelt. - Perform a test of the hydraulic functions including lift, tilt, and auxiliary hydraulics in a clear area. - Operate at a safe speed and maintain exclusion zones around people, powerlines, and other hazards (use spotters if necessary). - Beware of uneven or sloping ground. - Use the skid-steer’s bucket or attachment cautiously to load, move, or grade materials as required. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Remove any debris from the bucket or other attachments used during operation. - Park the skid-steer in a safe and designated area, lowering any equipped attachments to the ground. - Perform a visual inspection for new signs of wear or damage after use. - Shut down the engine and remove the keys to help prevent unauthorised access to the machine. - Record the machine’s operational hours and any issues experienced during use in the appropriate logs. - Employ any necessary lockout/tagout procedures and ensure the skid-steer is secured from theft or vandalism. - Notify maintenance personnel of any problems noted during operation or apparent maintenance needs. |  |  |  |  |  |  |