| TASK | Soldering Iron |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☒ | Electricity | ☒ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☐ |
|  | Pinch / crush | ☐ | Noise / vibration | ☐ | Plant interaction | ☐ |
|  | High pressure | ☐ | Other: | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Inspect the soldering iron for visible damage, wear, or defects. - Check the power cord for cuts, frays, or any sign of wear. - Ensure the soldering iron stand is stable and in good condition. - Verify the tip of the soldering iron is clean and properly attached. - Choose the correct tip for your soldering needs. - Set up your work area in a well-ventilated space, free from flammable materials. - Prepare all materials to be soldered (e.g., wires) ensuring they are clean. - Use a damp sponge or brass tip cleaner to clean the tip. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Plug in the soldering iron and allow it to heat up to the desired temperature. - Use the damp sponge or brass tip cleaner to clean the tip again once hot. - Apply a small amount of solder to the tip to tin it, which will help improve heat transfer. - Hold the soldering iron like a pen, maintaining a comfortable grip. - Heat the joint to be soldered with the tip, and then apply solder directly to the joint, not the tip of the iron. - Let the solder flow around the joint and remove the iron while keeping the solder in place. - Avoid moving the components until the solder has cooled and solidified. - Clean the tip with a damp sponge or brass tip cleaner as needed. - Turn off the soldering iron when not actively soldering to prevent overheating and unnecessary wear on the tip. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off the soldering iron immediately after use and unplug it. - Clean the tip with the damp sponge or brass tip cleaner, and then apply a small amount of solder to 'tin' the tip before storage. - Allow the soldering iron to cool completely before handling further or attempting to store it. - Once cool, store the soldering iron in a safe, dry place, preferably in a holder to prevent the tip from encountering surfaces that could be damaged. - Report any defects or problems noted during use or inspection. - Document the maintenance and usage log, if applicable, for future reference and compliance. |  |  |  |  |  |  |