| TASK | Table Saw |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☒ |
|  | Cuts / laceration | ☒ | Dust | ☒ | Rollover | ☐ |
|  | Pinch / crush | ☐ | Noise / vibration | ☒ | Plant interaction | ☐ |
|  | High pressure | ☐ | Kickback | ☒ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Ensure all personnel are trained and authorized to operate the table saw. - Inspect the table saw for any visible damage, loose components, or signs of wear. - Check that the saw blade is sharp, properly aligned, and securely fastened. - Verify that the fence and miter gauge are properly adjusted and securely locked in place. - Ensure that the power cord is in good condition and that the power switch is functioning correctly. - Confirm that fire extinguishing equipment is readily available in case of emergency. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Wear appropriate personal protective equipment (PPE) including safety glasses, hearing protection, and closed-toe shoes. - Position the workpiece on the table saw, ensuring that it is properly supported and clear of the blade path. - Adjust the blade height and angle as necessary for the desired cut. - Stand to the side of the blade, not directly in line with it, and keep hands clear of the blade path. - Start the table saw and allow it to reach full speed before making any cuts. - Feed the workpiece into the blade with a steady, controlled motion, keeping fingers away from the blade. - Use push sticks or other tools to guide small or narrow workpieces through the cut. - Never reach over or behind the blade while it is running, and never remove cut-off pieces until the blade has stopped completely. - If any issues or errors occur during operation, stop the table saw immediately and troubleshoot the problem before resuming. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Turn off the table saw and wait for the blade to come to a complete stop before removing the workpiece. - Inspect the cut and the table saw for any defects or issues, reporting any problems to the appropriate personnel. - Clean up the work area, removing any sawdust or debris from the table saw and surrounding area. - Store the table saw accessories and tools in a designated area, ensuring they are protected from damage and unauthorized use. |  |  |  |  |  |  |