| TASK | Telehandler |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☒ |
|  | Pinch / crush | ☐ | Noise / vibration | ☐ | Plant interaction | ☒ |
|  | High pressure | ☐ | Kickback | ☐ | Other: | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Check the telehandler for any visible damage, ensuring tires are inflated and free of damage. - Complete all other pre-start checks as per manufacturer’s guidelines. - Ensure all guards and covers are in place and secure. - Review the load chart to understand the weight limits and boom extension restrictions. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Ensure personnel are competent and authorised. - Ensure personnel are fit for duty and not impaired by drugs or alcohol. - Do not use a mobile phone while operating. - Wear your seatbelt while operating. - Operate at a safe speed and maintain exclusion zones around people, powerlines, and other hazards (use spotters if necessary). - Use smooth and steady movements when lifting, extending, or retracting the boom. - Keep the load low to the ground while transporting and raise only as necessary for placement. - Beware of uneven or sloping ground. - Never carry people or lift loads over people. - Use only suitable lifting gear and attachments. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Ensure that the boom is fully retracted and lowered and that any attachments are securely placed on the ground. - Conduct a post-operation inspection for any new damage or issues. - Remove any debris from the telehandler that could become a hazard. - Turn off the engine, remove the key, and secure the vehicle to prevent unauthorised use. - Document and report any problems or incidents that occurred during operation. |  |  |  |  |  |  |