| TASK | Trencher |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☐ | Dust | ☒ | Rollover | ☒ |
|  | Pinch / crush | ☒ | Noise / vibration | ☒ | Plant interaction | ☒ |
|  | High pressure | ☐ | Other: | ☐ | Chemical | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Walk around the trencher to inspect for any damage to the body, chain, teeth, and undercarriage. - Check if the controls, including levers and switches, are functioning and responsive. - Ensure the hydraulic fluid levels are sufficient and verify there are no leaks. - Exam the conveyor belt, if applicable, for wear and tear, and ensure that it operates smoothly. - Confirm that all safety devices, such as seat belts and roll-over protection structures, are present and in working order. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Adjust the operator’s seat and controls to reach them comfortably before starting the trencher. - Start the trencher and observe the operation of hydraulics and conveyor systems for any irregularities. - Operate the trencher at a consistent and safe speed, taking care to make slow, controlled turns and movements. - Always be aware of the boom location and trench depth to avoid underground utilities and obstructions. - Never leave the trencher unattended while it is running and always be mindful of bystanders or crew on the ground. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Lower the digging boom and ensure the trencher is turned off before exiting the operator’s station. - Conduct a post-use inspection for any damages or issues that may have arisen during operation. - Clean off any dirt and debris from the trencher to prevent buildup and maintain the equipment. - Document the hours of operation, fuel used, and note any maintenance requirements or incidents that occurred during use. - Secure the trencher by parking it in a safe location and removing the keys from the ignition. |  |  |  |  |  |  |