| TASK | Vac Truck |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☒ |
|  | Pinch / crush | ☒ | Noise / vibration | ☒ | Plant interaction | ☒ |
|  | High pressure | ☐ | Other: | ☐ | Chemical | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Inspect the tank for potential leaks or damage. - Check the integrity and attachment of hoses, ensuring they are clear of blockages. - Conduct all pre-start checks as per manufacturer’s guidelines. - Look over the PTO (Power Take-Off) system to ensure it is engaging properly. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Complete all pre-start checks as per manufacturer’s guidelines. - Ensure personnel are competent and authorized. - Ensure personnel are fit for duty and not impaired by drugs or alcohol. - Do not use a mobile phone while operating and always wear your seatbelt. - Start the engine and allow all systems to warm up. - Engage the PTO to start the vacuum pump and test suction briefly to confirm proper operation. - Operate at a safe speed and maintain exclusion zones around people, powerlines, and other hazards (use spotters if necessary). - Beware of uneven or sloping ground. - When arriving at the job site, position the truck appropriately and set any required safety signals or barriers. - Deploy hoses carefully to the excavation or collection point. - Operate the vacuum system, monitoring pressure gauges and other controls to maintain safe and efficient performance. - Keep a watchful eye on the tank level to prevent overfilling and potential spills. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Properly clean and stow hoses and any other equipment used. - Disengage the PTO and vacuum systems, ensuring the vehicle is ready for normal driving. - Document the completed job, recording any issues and the volume of materials collected, if necessary. - Conduct a post-use inspection of the vehicle, including the vacuum system, for maintenance needs. - Park the vehicle in a secure area, lock all doors, and remove the keys to prevent unauthorised access. - Report to the appropriate personnel for any necessary offloading of collected materials following environmental regulations. |  |  |  |  |  |  |