| TASK | Water Truck |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HAZARDS | Flying debris | ☐ | Heat / cold | ☐ | Electricity | ☐ |
|  | Cuts / laceration | ☐ | Dust | ☐ | Rollover | ☒ |
|  | Pinch / crush | ☒ | Noise / vibration | ☒ | Plant interaction | ☒ |
|  | High pressure | ☐ | Other: | ☐ | Chemical | ☐ |
| PPE REQUIRED |  |  |  |  |  |  |
| PRE-START CHECKS |  |  |  |  |  |  |
| - Conduct all pre-start checks as per manufacturer’s guidelines. - Check the spray heads and nozzles to ensure they are clear and operational. - Verify that hoses, pumps, and valves are free of damage, are properly connected and in working order. - Confirm that the cabin's gauges and control systems are working correctly. - Check that all safety equipment, such as fire extinguishers and warning triangles, are present and in working order. - Adjust the driver’s seat and mirrors for comfort and ensure you have clear visibility. |  |  |  |  |  |  |
| SAFE OPERATING PROCEDURE |  |  |  |  |  |  |
| - Ensure personnel are competent and authorised. - Ensure personnel are fit for duty and not impaired by drugs or alcohol. - Do not use a mobile phone while operating and always wear your seatbelt. - Start the truck and let it idle to check for unusual engine noises or warning lights. - Drive to road conditions, obeying all traffic laws. - Ensure appropriate allowances are made when the tank is full as the shifting weight can significantly affect handling and braking distances. - Operate at a safe speed and maintain exclusion zones around people, powerlines, and other hazards (use spotters if necessary). - Do not spray on or near people or animals. - Operate the water distribution controls smoothly to avoid sudden weight shifts that may lead to loss of control or vehicle rollover. - Beware of uneven or sloping ground. |  |  |  |  |  |  |
| POST-OPERATION PROCEDURE |  |  |  |  |  |  |
| - Shut down water pump system. Ensure spray heads and nozzles are turned off. - Inspect the truck again for any new signs of wear or damage, particularly checking the spray system for clogs or debris. - Log any mechanical issues observed during the day’s operation, and report these to the maintenance team or supervisor. - Clean out any debris from the water tank's filters and screens. - Park the truck in the designated area and ensure the vehicle is properly secured. - Turn off the ignition, remove the keys, and lock the doors. - Complete any necessary documentation such as logs for water used, distance covered, and any incidents that may have occurred. |  |  |  |  |  |  |